1. Technician A says that diesel engines require a low-pressure fuel supply to transfer fuel to the high-pressure system. Technician B says the engine needs pressure and volume from the low-pressure fuel system for the engine to function correctly. Who is correct? a.; Technician A. b.; Technician B. c.; Both Technician A and Technician B. d.; Neither Technician A nor Technician B. ANSWER:; c. POINTS:; 100. REFERENCES:; 435. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:34 AM. DATE MODIFIED:; 11/15/2018 4:15 PM.

2. All of the following are functions of the low-pressure fuel system Except? a.; Store and supply an adequate volume of clean fuel. b.; Prevent wax gelling. c.; Regulate fuel temperature. d.; Inject fuel into the combustion chamber.. ANSWER:; d. POINTS:; 100. REFERENCES:; 436. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:34 AM. DATE MODIFIED:; 3/8/2019 6:57 PM.

3. Technician A says that low diesel fuel temperature causes problems, but high fuel temperatures are not a problem. Technician B says that both low and high fuel temperature can cause problems with diesel fuel. Who is correct? a.; Technician A. b.; Technician B. c.; Both Technician A and Technician B. d.; Neither Technician A nor Technician B. ANSWER:; b. POINTS:; 100. REFERENCES:; 436. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:34 AM. DATE MODIFIED:; 11/15/2018 4:15 PM.

4. Technician A says that a function of the fuel tank is to store an adequate supply of fuel. Technician B says that a secondary function of the fuel tank is to allow fuel returned from the engine to cool after absorbing heat from the engine and fuel injectors. Who is correct? a.; Technician A. b.; Technician B. c.; Both Technician A and Technician B. d.; Neither Technician A nor Technician B. ANSWER:; c. POINTS:; 100. REFERENCES:; 437. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:34 AM. DATE MODIFIED:; 11/15/2018 4:15 PM.

5. Which of the following statements is correct concerning fuel tank ventilation? a.; Fuel tanks require some venting to the atmosphere.. b.; Fuel tanks should be completely sealed.. c.; Both A and B. d.; Neither A nor B. ANSWER:; a. POINTS:; 100. REFERENCES:; 437. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:34 AM. DATE MODIFIED:; 11/15/2018 4:15 PM.

6. Technician A says that low-pressure fuel lines are always constructed of steel. Technician B says that low-pressure fuel lines can be made from braided steel, corrosion-resistant metal tubes, and diesel fuel-compatible rubber material. Who is correct? a.; Technician A. b.; Technician B. c.; Both Technician A and Technician B. d.; Neither Technician A nor Technician B. ANSWER:; b. POINTS:; 100. REFERENCES:; 439. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:34 AM. DATE MODIFIED:; 11/15/2018 4:15 PM.

7. Technician A says that fuel tank pickups almost touch the bottom of the tank. Technician B says that the fuel tank pickup is designed to prevent the accumulation of contaminants. Who is correct? ​ a.; Technician A. b.; Technician B. c.; Both Technician A and Technician B. d.; Neither Technician A nor Technician B. ANSWER:; c. POINTS:; 100. REFERENCES:; 439. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:34 AM. DATE MODIFIED:; 3/8/2019 7:00 PM.

8. Technician A says that diesel fuel systems do not require fuel return circuits. Technician B says that almost all diesel fuel systems have fuel return circuits to allow fuel to circulate through the injectors and other fuel system components to remove vapor and lubricate and cool fuel system components. Who is correct? a.; Technician A. b.; Technician B. c.; Both Technician A and Technician B. d.; Neither Technician A nor Technician B. ANSWER:; b. POINTS:; 100. REFERENCES:; 440. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:34 AM. DATE MODIFIED:; 11/15/2018 4:15 PM.

9. Which of the following statements is correct concerning fuel tank sending units? a.; A resistive fuel sending unit completes the circuit from the gauge to ground.. b.; A resistive fuel sending unit completes the circuit from the gauge to power.. c.; Both A and B. d.; Neither A nor B. ANSWER:; a. POINTS:; 100. REFERENCES:; 441. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:34 AM. DATE MODIFIED:; 11/15/2018 4:15 PM.

10. Technician A says that damage causing contamination can usually be viewed with a casual inspection. Technician B says that most wear from containments occurs if they are larger than 10 microns. Who is correct? a.; Technician A. b.; Technician B. c.; Both Technician A and Technician B. d.; Neither Technician A nor Technician B. ANSWER:; d. POINTS:; 100. REFERENCES:; 442. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:34 AM. DATE MODIFIED:; 11/15/2018 4:15 PM.

11. Technician A says smaller particles of asphaltenes can produce abrasive damage in the fuel system. Technician B says that although large asphaltenes are considered harmless to the injection system components, they do plug fuel filters. Who is correct? a.; Technician A. b.; Technician B. c.; Both Technician A and Technician B. d.; Neither Technician A nor Technician B. ANSWER:; c. POINTS:; 100. REFERENCES:; 443. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:34 AM. DATE MODIFIED:; 11/15/2018 4:15 PM.

12. Technician A says that a specialized fuel filter and fuel-water separator is used to remove water from fuel before it does any damage. Technician B says that the normal fuel filters are adequate to remove water from the fuel system. Who is correct? a.; Technician A. b.; Technician B. c.; Both Technician A and Technician B. d.; Neither Technician A nor Technician B. ANSWER:; a. POINTS:; 100. REFERENCES:; 444. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:34 AM. DATE MODIFIED:; 11/15/2018 4:15 PM.

13. Which of the following statements is correct concerning bacteria and fungi in the fuel tank? a.; Bacteria and fungi are filtered from the fuel by the fuel filters.. b.; Bacteria and fungi can multiply throughout the system and clog fuel filters.. c.; A biocide is recognized as the best means to eliminate bacteria and fungi.. d.; All of these are correct.. ANSWER:; d. POINTS:; 100. REFERENCES:; 446. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:34 AM. DATE MODIFIED:; 11/15/2018 4:15 PM.

14. Which of the following statements is correct concerning wax or paraffin in fuel? a.; Wax is normally dissolved in the fuel as liquid.. b.; In cold weather, wax begins to congeal and may plug the filter.. c.; Both A and B. d.; Neither A nor B. ANSWER:; c. POINTS:; 100. REFERENCES:; 446. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:34 AM. DATE MODIFIED:; 11/15/2018 4:15 PM.

15. Technician A says a 5 μ filter operating at 50% efficiency will only trap half the particles 5 μ and larger. Technician B says filters are classified by filtration media and by efficiency. Who is correct? a.; Technician A. b.; Technician B. c.; Both Technician A and Technician B. d.; Neither Technician A nor Technician B. ANSWER:; c. POINTS:; 100. REFERENCES:; 447. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:34 AM. DATE MODIFIED:; 11/15/2018 4:15 PM.

16. Which of the following statements is correct? a.; The primary filter is the last filter in the fuel stream after the tank.. b.; The primary filter is the first filter in the fuel stream after the tank.. c.; Both A and B. d.; Neither A nor B. ANSWER:; b. POINTS:; 100. REFERENCES:; 447. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:34 AM. DATE MODIFIED:; 11/15/2018 4:15 PM.

17. Technician A says that most cartridge-type fuel filter housings are designed to prevent the fuel system from operating without a fuel filter. Technician B says that cartridge-type filters have become far more popular with manufacturers. Who is correct? a.; Technician A. b.; Technician B. c.; Both Technician A and Technician B. d.; Neither Technician A nor Technician B. ANSWER:; c. POINTS:; 100. REFERENCES:; 448. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:34 AM. DATE MODIFIED:; 11/15/2018 4:15 PM.

18. Technician A says that fuel-water separators, water-in-fuel sensors, fuel heaters, and filters are often integrated into a single compact unit. Technician B says that water-in-fuel (WIF) sensors use an electrical conductivity probe to sense a specific level of water in fuel and cause a dash-mounted light to illuminate. Who is correct? a.; Technician A. b.; Technician B. c.; Both Technician A and Technician B. d.; Neither Technician A nor Technician B. ANSWER:; c. POINTS:; 100. REFERENCES:; 449. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:34 AM. DATE MODIFIED:; 11/15/2018 4:15 PM.

19. Technician A says that a positive displacement pump output is not dependent on engine speed. Technician B says that a gear pump is a positive displacement pump. Who is correct? a.; Technician A. b.; Technician B. c.; Both Technician A and Technician B. d.; Neither Technician A nor Technician B. ANSWER:; b. POINTS:; 100. REFERENCES:; 450. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:34 AM. DATE MODIFIED:; 11/15/2018 4:15 PM.

20. Which of the following statements is correct? a.; Piston pumps operate at higher pressure and with greater durability than diaphragm pumps.. b.; As outlet pressure increases in piston pumps, there is more piston movement.. c.; Both A and B. d.; Neither A nor B. ANSWER:; a. POINTS:; 100. REFERENCES:; 452. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:34 AM. DATE MODIFIED:; 11/15/2018 4:15 PM.

21. Technician A says that an alternative to using a hand primer is to use fuel from a can or other apparatus to fill new fuel filters. Technician B says that using a hand primer to fill new fuel filters minimizes the possibility of dirt contamination of the high-pressure fuel. Who is correct? a.; Technician A. b.; Technician B. c.; Both Technician A and Technician B. d.; Neither Technician A nor Technician B. ANSWER:; b. POINTS:; 100. REFERENCES:; 453. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:34 AM. DATE MODIFIED:; 11/15/2018 4:15 PM.

22. Technician A says that the Detroit FSIC test includes an evaluation of the low-pressure fuel system. Technician B says that the Detroit FSIC test only test the high-pressure fuel system. Who is correct? a.; Technician A. b.; Technician B. c.; Both Technician A and Technician B. d.; Neither Technician A nor Technician B. ANSWER:; a. POINTS:; 100. REFERENCES:; 455. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:34 AM. DATE MODIFIED:; 11/15/2018 4:15 PM.

23. Which of the following statements is correct regarding the possible result of an air leak on suction lines? a.; Hard starting. b.; Low power. c.; Both A and B. d.; Neither A nor B. ANSWER:; c. POINTS:; 100. REFERENCES:; 456. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:34 AM. DATE MODIFIED:; 11/15/2018 4:15 PM.

24. Which of the following statements is correct in relation to filter service? a.; Fuel filters have a bypass valve in case they become restricted.. b.; Fuel filters have no bypass valve when they become restricted; consequently, fuel flow decreases.. c.; Both A and B. d.; Neither A nor B. ANSWER:; b. POINTS:; 100. REFERENCES:; 456. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:34 AM. DATE MODIFIED:; 11/15/2018 4:15 PM.

25. Technician A says that restriction occurs across the fuel filter when it begins to load with contaminants. Technician B says that filter contamination can be check by connecting a vacuum gauge with a T-fitting or service port between the transfer pump and primary filter. Who is correct? a.; Technician A. b.; Technician B. c.; Both Technician A and Technician B. d.; Neither Technician A nor Technician B. ANSWER:; c. POINTS:; 100. REFERENCES:; 456. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:34 AM. DATE MODIFIED:; 11/15/2018 4:15 PM.

26. Technician A says that a restriction measured at the outlet of a primary filter can be caused by a plugged filter or a restriction between the filter and the fuel tank. Technician B says that a primary filter restriction is always caused by a plugged filter. Who is correct? ​ a.; Technician A. b.; Technician B. c.; Both Technician A and Technician B. d.; Neither Technician A nor Technician B. ANSWER:; a. POINTS:; 100. REFERENCES:; 457. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:34 AM. DATE MODIFIED:; 3/8/2019 7:02 PM.

27. Technician A says that primary filters operate at higher pressure than secondary filters. Technician B says that secondary filters operate at higher pressure than primary filters. Who is correct? a.; Technician A. b.; Technician B. c.; Both Technician A and Technician B. d.; Neither Technician A nor Technician B. ANSWER:; b. POINTS:; 100. REFERENCES:; 457. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:34 AM. DATE MODIFIED:; 11/15/2018 4:15 PM.

28. Technician A says that oil contamination of fuel can be caused by damaged seals in gear-type fuel transfer pump. Technician B says it can be caused by failed O-rings in hydraulically actuated electronic unit injection (HEUI) injectors. Who is correct? a.; Technician A. b.; Technician B. c.; Both Technician A and Technician B. d.; Neither Technician A nor Technician B. ANSWER:; c. POINTS:; 100. REFERENCES:; 459. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:34 AM. DATE MODIFIED:; 11/15/2018 4:15 PM.

29. Which of the following statements is correct with regard to priming fuel systems? a.; A fuel system should be primed whenever fuel filters are replaced.. b.; Fuel systems do not need to be primed on modern diesel engines.. c.; Both A and B. d.; Neither A nor B. ANSWER:; a. POINTS:; 100. REFERENCES:; 460. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:34 AM. DATE MODIFIED:; 11/15/2018 4:15 PM.

30. Technician A says that any air in the fuel system will prevent the fuel from properly pressurizing. Technician B says that if there is air in the fuel system, the engine will quit or not start at all. Who is correct? a.; Technician A. b.; Technician B. c.; Both Technician A and Technician B. d.; Neither Technician A nor Technician B. ANSWER:; c. POINTS:; 100. REFERENCES:; 460. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:34 AM. DATE MODIFIED:; 11/15/2018 4:15 PM.

31. What percentage of fuel loss is associated with the increase of diesel fuel temperature in 10°F increments above 115°F? a.; 5%. b.; 4%. c.; 3%. d.; 2%. ANSWER:; d. POINTS:; 1. REFERENCES:; 437. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:23 AM. DATE MODIFIED:; 11/15/2018 4:15 PM.

32. Materials that should not be used to manufacture fuel tanks include, but are not limited to, __________. a.; Steel alloy. b.; Plastics. c.; Galvanized steels. d.; All of these are correct.. ANSWER:; c. POINTS:; 1. REFERENCES:; 437. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:23 AM. DATE MODIFIED:; 3/8/2019 6:26 PM.

33. On highway, diesel fuel tanks __________. a.; Are pressurized. b.; Are vented. c.; Feature double-walled construction. d.; None of these is correct.. ANSWER:; b. POINTS:; 1. REFERENCES:; 429. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:23 AM. DATE MODIFIED:; 11/15/2018 4:15 PM.

34. If a fuel tank vent is incorporated within the fuel cap, what is the release pressure set to? a.; 1 psi. b.; 2 psi. c.; 3 psi. d.; 4 psi. ANSWER:; b. POINTS:; 1. REFERENCES:; 438. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:23 AM. DATE MODIFIED:; 11/15/2018 4:15 PM.

35. At what temperature is a thermal relief valve set? a.; 200°F. b.; 225°F. c.; 250°F. d.; 275°F. ANSWER:; b. POINTS:; 1. REFERENCES:; 438. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:23 AM. DATE MODIFIED:; 11/15/2018 4:15 PM.

36. A vacuum valve in a fuel cap lets __________. a.; Fuel out. b.; Air out. c.; Air in. d.; Fuel in. ANSWER:; c. POINTS:; 1. REFERENCES:; 438-439. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:23 AM. DATE MODIFIED:; 11/15/2018 4:15 PM.

37. Diesel low-pressure fuel lines must not be routed __________. a.; Through the frame. b.; Through the cab interior. c.; Through the wiring harness. d.; Below the fuel filter inlet. ANSWER:; b. POINTS:; 1. REFERENCES:; 439. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:23 AM. DATE MODIFIED:; 11/15/2018 4:15 PM.

38. A fuel module may include a __________. a.; Filter. b.; Hand primer pump. c.; Fuel heater. d.; All of these are correct.. ANSWER:; d. POINTS:; 1. REFERENCES:; 446. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:23 AM. DATE MODIFIED:; 11/15/2018 4:15 PM.

39. A fuel tank pickup tube works by means of __________. a.; Gravity. b.; Pressure. c.; A Venturi effect. d.; Head pressure. ANSWER:; c. POINTS:; 1. REFERENCES:; 439. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:23 AM. DATE MODIFIED:; 11/15/2018 4:15 PM.

40. What is the function of a return flow splitter valve? a.; Prevents air in fuel. b.; Eliminates crossover line. c.; Equality divided flow to tanks. d.; All of these are correct.. ANSWER:; a. POINTS:; 1. REFERENCES:; 439. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:23 AM. DATE MODIFIED:; 11/15/2018 4:15 PM.

41. A return flow splitter valve regulates _________ ? a.; Fuel tank pressure. b.; Fuel return flow. c.; Fuel pump flow. d.; Fuel pump pressure. ANSWER:; b. POINTS:; 1. REFERENCES:; 440. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:23 AM. DATE MODIFIED:; 4/3/2019 6:25 PM.

42. Current to the fuel tank sending unit is supplied from the __________. a.; Battery. b.; Fuel gauge module. c.; Ignition switch. d.; Fuel pump. ANSWER:; b. POINTS:; 1. REFERENCES:; 441. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:23 AM. DATE MODIFIED:; 11/15/2018 4:15 PM.

43. What does high resistance from the fuel sender’s float mechanism indicate? a.; Empty tanks. b.; Full tanks. c.; Not reading level. d.; None of these is correct.. ANSWER:; b. POINTS:; 1. REFERENCES:; 441. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:23 AM. DATE MODIFIED:; 11/15/2018 4:15 PM.

44. A body controller incorporated into the fuel sending circuit may utilize a pull up resister set to a value of __________. a.; 12 OHMS. b.; 12 kOHMS. c.; 1.2 kOHMS. d.; 120 kOHMS. ANSWER:; c. POINTS:; 1. REFERENCES:; 442. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:23 AM. DATE MODIFIED:; 11/15/2018 4:15 PM.

45. What is the most destructive contaminate within a fuel? a.; Asphaltenes. b.; Dirt. c.; Water. d.; Gel. ANSWER:; c. POINTS:; 1. REFERENCES:; 443. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:23 AM. DATE MODIFIED:; 11/15/2018 4:15 PM.

46. Humbugs feed on __________. a.; Water. b.; HC. c.; Alcohol. d.; All of these are correct.. ANSWER:; b. POINTS:; 1. REFERENCES:; 446. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:23 AM. DATE MODIFIED:; 11/15/2018 4:15 PM.

47. Most diesel engines will circulate __________ gallons/hour. a.; 100. b.; 200. c.; 300. d.; 400. ANSWER:; c. POINTS:; 1. REFERENCES:; 446. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:23 AM. DATE MODIFIED:; 11/15/2018 4:15 PM.

48. Fuel heaters are often located within the __________. a.; Fuel filter. b.; Fuel hand primer. c.; Fuel transfer pump. d.; Injectors. ANSWER:; a. POINTS:; 1. REFERENCES:; 449. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:23 AM. DATE MODIFIED:; 3/8/2019 6:42 PM.

49. The purpose of the primary fuel filter(s) is to protect the __________. a.; Fuel tank. b.; Fuel pump. c.; Injectors. d.; Return check valve. ANSWER:; b. POINTS:; 1. REFERENCES:; 447. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:23 AM. DATE MODIFIED:; 11/15/2018 4:15 PM.

50. Primary fuel efficiency is 95–98% at __________. a.; 2–5 microns. b.; 5–10 microns. c.; 10–100 microns. d.; 50–100 microns. ANSWER:; c. POINTS:; 1. REFERENCES:; 447. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:23 AM. DATE MODIFIED:; 11/15/2018 4:15 PM.

51. Fuel system gear pumps are of the __________ design. a.; Gravity. b.; Nonpositive displacement. c.; Positive displacement. d.; Return flow. ANSWER:; c. POINTS:; 1. REFERENCES:; 449. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:23 AM. DATE MODIFIED:; 3/8/2019 6:43 PM.

52. Pressure of fuel delivery out of a positive displacement pump increases as __________. a.; Filter restriction increases. b.; Engine RPM increases. c.; Engine RPM decreases. d.; Road speed of vehicle increases. ANSWER:; b. POINTS:; 1. REFERENCES:; 450. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:23 AM. DATE MODIFIED:; 11/15/2018 4:15 PM.

53. A fuel return check valve __________. a.; Limits fuel pressure. b.; Prevents drain back. c.; Flows fuel to the fuel tank. d.; All of these are correct.. ANSWER:; d. POINTS:; 1. REFERENCES:; 450. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:23 AM. DATE MODIFIED:; 11/15/2018 4:15 PM.

54. A mechanical diaphragm pump will produce pressures around __________. a.; 10 pounds per psi. b.; 5 pounds per psi. c.; 7 pounds per psi. d.; 4 pounds per psi. ANSWER:; c. POINTS:; 1. REFERENCES:; 451. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:23 AM. DATE MODIFIED:; 11/15/2018 4:15 PM.

55. The split linkage of a mechanical diaphragm pump operates on a(n) __________. a.; Eccentric cam lobe. b.; Diaphragm. c.; Inlet valve. d.; Outlet valve. ANSWER:; a. POINTS:; 1. REFERENCES:; 451. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:23 AM. DATE MODIFIED:; 11/15/2018 4:15 PM.

56. Which component connects the split linkage to the diaphragm in a mechanical diaphragm pump? a.; Eccentric. b.; Pull rod. c.; Engine oil seal. d.; Pressure spring. ANSWER:; b. POINTS:; 1. REFERENCES:; 451. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:23 AM. DATE MODIFIED:; 11/15/2018 4:15 PM.

57. Technician A says that deteriorated check valves can effect outlet pressure of a mechanical diaphragm pump. Technician B says the diaphragm pumps are not used in diesel engine fuel systems. Who is correct? a.; Technician A. b.; Technician B. c.; Both Technician A and Technician B. d.; Neither Technician A nor Technician B. ANSWER:; a. POINTS:; 1. REFERENCES:; 458. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:23 AM. DATE MODIFIED:; 3/8/2019 6:49 PM.

58. The diaphragm mechanical pump is activated by the __________. a.; Engine camshaft. b.; Engine crankshaft. c.; Rocker lever. d.; Electric stepper motor. ANSWER:; a. POINTS:; 1. REFERENCES:; 451. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:23 AM. DATE MODIFIED:; 11/15/2018 4:15 PM.

59. Roller vane pumps are normally activated __________. a.; By a camshaft. b.; Electrically. c.; By a rocker system. d.; None of these is correct.. ANSWER:; b. POINTS:; 1. REFERENCES:; 453. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:23 AM. DATE MODIFIED:; 11/15/2018 4:15 PM.

60. Where are mechanical vane-type pumps usually located? a.; Fuel tank. b.; Frame. c.; Injection pump. d.; Engine block. ANSWER:; c. POINTS:; 1. REFERENCES:; 454. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:23 AM. DATE MODIFIED:; 3/8/2019 6:49 PM.

61. A fuel system sight glass is used for __________. a.; Measuring restriction. b.; Measuring pressure. c.; Monitoring for air. d.; Calibrating injector volume. ANSWER:; c. POINTS:; 1. REFERENCES:; 454. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:23 AM. DATE MODIFIED:; 11/15/2018 4:15 PM.

62. Fuel volume can be measured with a __________. a.; Restriction gauge. b.; Pressure gauge. c.; Sight glass. d.; Graduated cylinder. ANSWER:; d. POINTS:; 1. REFERENCES:; 454. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:23 AM. DATE MODIFIED:; 11/15/2018 4:15 PM.

63. What type of filter has a bypass valve? a.; Oil filter. b.; Fuel primary filter. c.; Fuel secondary filter. d.; Air filter. ANSWER:; a. POINTS:; 1. REFERENCES:; 456. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:23 AM. DATE MODIFIED:; 11/15/2018 4:15 PM.

64. Generally, fuel filter restriction should not be more than __________ inches/Hg, measured at high idle. a.; 5. b.; 7. c.; 12. d.; 15. ANSWER:; c. POINTS:; 1. REFERENCES:; 456. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:23 AM. DATE MODIFIED:; 11/15/2018 4:15 PM.

65. Which is the recommended tool when performing primary fuel system inspection? a.; Restriction gauge. b.; Pressure differential gauge. c.; Sight glass. d.; Pressure gauge. ANSWER:; a. POINTS:; 1. REFERENCES:; 456. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:23 AM. DATE MODIFIED:; 11/15/2018 4:15 PM.

66. To measure fuel pump outlet pressure, where is the pressure gauge installed? a.; After the return check valve. b.; Outlet of the secondary filter. c.; After the primary filter. d.; At the fuel return line at tank. ANSWER:; b. POINTS:; 1. REFERENCES:; 461. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:23 AM. DATE MODIFIED:; 11/15/2018 4:15 PM.

67. What can cause a fuel in oil condition? a.; Failed injector. b.; Damaged fuel pump seals. c.; Injector O-ring failure. d.; All of these are correct.. ANSWER:; d. POINTS:; 1. REFERENCES:; 459. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:23 AM. DATE MODIFIED:; 11/15/2018 4:15 PM.

68. A loose primary side fuel fitting will __________. a.; Leak fuel. b.; Draw air. c.; Have no effect. d.; None of these is correct.. ANSWER:; b. POINTS:; 1. REFERENCES:; 456, 457. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:23 AM. DATE MODIFIED:; 11/15/2018 4:15 PM.

69. Which acronym is associated with Detroit Diesels diagnostic software system? a.; DDDL. b.; FSIC. c.; DDFS. d.; DFSIC. ANSWER:; a. POINTS:; 1. REFERENCES:; 455. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:23 AM. DATE MODIFIED:; 11/15/2018 4:15 PM.

70. A hand primer pump functions __________ the high-pressure gear fuel pump. a.; In line with. b.; Parallel to. c.; Before. d.; After. ANSWER:; b. POINTS:; 1. REFERENCES:; 452. QUESTION TYPE:; Multiple Choice. HAS VARIABLES:; False. DATE CREATED:; 3/7/2018 8:23 AM. DATE MODIFIED:; 11/15/2018 4:15 PM.